List of Trial Types Carried Out by

the Laboratory of Trials and Metrology of RPA ‘Mars’

| Types of Trials for Exposure to External Factors | Normative Documents Specifying Trial Requirements | Ranges for Characteristics of External Factors Generated by Testing Equipment | Weight and Dimensions of Products to Be Tested, kg; mmmmmm |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| 1 Mechanical Trials | GOST RV 20.39.304-98, specifications for products, trial programmes and procedures, GOST RV 20.57.305-98 |  |  |
| 1.1 Trials for exposure to vibrations |  | acceleration: 102 g; frequency span: from 5 Hz to 2,000 Hz | 950; 1,200×1,200×1,200 |
| 1.1.1 Trials to define critical frequencies | GOST RV 20.57.305-98 | acceleration: 102 g; frequency span: from 5 Hz to 2,000 Hz | 950; 1,200×1,200×1,200 |
| 1.1.2 Trials for stability in case of vibration exposure | GOST RV 20.57.305-98 | acceleration: 102 g; frequency span: from 5 Hz to 2,000 Hz | 950; 1,200×1,200×1,200 |
| 1.1.3 Trials for strength in case of vibration exposure | GOST RV 20.57.305-98 | acceleration: 102 g; frequency span: from 5 Hz to 2,000 Hz | 950; 1,200×1,200×1,200 |
| 1.1.4 Trials for exposure to sinusoidal vibration (one frequency) | GOST RV 20.57.305-98 | acceleration: 102 g; frequency span: from 5 Hz to 2,000 Hz | 950; 1,200×1,200×1,200 |
| 1.2 Trials for exposure to mechanical shocks |  | acceleration: 1,000 g; shock-pulse duration: up to 30 ms; shock-repetition frequency: up to 3 Hz | 450; 600×800×2,000 |
| 1.2.1 Trials for stability in case of bump exposure | GOST RV 20.57.305-98 | acceleration: 1,000 g; shock-pulse duration: up to 30 ms; shock-repetition frequency: up to 3 Hz | 450; 600×800×2,000 |
| 1.2.2 Trials for strength in case of bump exposure | GOST RV 20.57.305-98 | acceleration: 1,000 g; shock-pulse duration: up to 30 ms; shock-repetition frequency: up to 3 Hz | 450; 600×800×2,000 |
| 1.2.3 Trials for strength and stability in case of shock exposure | GOST RV 20.57.305-98 | acceleration: 1,000 g; shock-pulse duration: up to 30 ms; shock-repetition frequency: up to 3 Hz | 450; 600×800×2,000 |
| 1.2.4 Trials for fastener-assembly strength in case of shock exposure | GOST RV 20.57.305-98 | acceleration: 1,000 g; shock-pulse duration: up to 30 ms; shock-repetition frequency: up to 3 Hz | 450; 600×800×2,000 |
| 1.2.5 Trials for strength in case of dropping | GOST RV 20.57.305-98 |  |  |
| 1.2.6 Trials for exposure to mechanical factors in case of transporting | GOST RV 20.57.305-98 | acceleration: 1,000 g; shock-pulse duration: up to 30 ms; shock-repetition frequency: up to 3 Hz | 450; 600×800×2,000 |
| 1.3 Trials for stability in case of pitching/rolling and permanent-tilt exposure | GOST RV 20.57.305-98 | tilt angle: ±45 degrees; period: from 7 s to 16 s | 1,000; 1,0501,0502,000 |
| 2 Environmental Trials | GOST RV 20.39.304-98, specifications for products, trial programmes and procedures, GOST RV 20.57.306-98, GOST RV 20.57.310-98 |  |  |
| 2.1 Trials for exposure to ambient higher temperature | GOST RV 20.57.306-98 | temperature: from – 75 ºС to 300 ºС | 1,000; 2,0502,3001,870 |
| 2.2 Trials for exposure to ambient lower temperature | GOST RV 20.57.306-98 | temperature: from – 75 ºС to 300 ºС | 1,000; 2,0502,3001,870 |
| 2.3 Trials for exposure to higher humidity | GOST RV 20.57.306-98 | temperature: from 10 ºС to 60 ºС; relative humidity: up to 100% | 1,000; 2,0502,3001,870 |
| 2.4 Trials for exposure to ambient-temperature change | GOST RV 20.57.306-98 | temperature: from – 75 ºС to 300 ºС | 1,000; 2,0502,3001,870 |
| 2.5 Trials for exposure to low atmospheric pressure | GOST RV 20.57.306-98 | up to 5 mm Hg | 1,000; 2,0502,3001,870 |
| 2.6 Trials for exposure to low atmospheric pressure in case of transporting by air | GOST RV 20.57.306-98 | up to 5 mm Hg; temperature: up to – 75 ºС | 1,000; 2,0502,3001,870 |
| 2.7 Trials for exposure to high air (gas) pressure | GOST RV 20.57.306-98 | up to 10 kgs/sq сm | 1,000; 1,0001,0001,000 |
| 2.8 Trials for exposure to condensed atmosphere precipitations (hoarfrost and dew) | GOST RV 20.57.306-98 | temperature: from – 75 ºС to 300 ºС | 1,000; 2,0502,3001,870 |
| 2.9 Trials for exposure to solar radiation | GOST RV 20.57.306-98 | density of IR-radiation flux: up to 1,200 W/sq m; density of UV-radiation flux: up to 68 W/sq m within from 0.32 µm to 0.4 µm; generation and maintenance of temperature (shade): 45 С | 1,000; 2,0502,3001,870 |
| 2.10 Trials for exposure to salt spray | GOST RV 20.57.306-98 | dispersion: from 1 µm to 10 µm; water content: from 2 g/m3 to 3 g/ m3; generation and maintenance of temperature: 35 С | 1,000; 1,7001,8001,700 |
| 2.11 Trials for leakproofness by immersion method | GOST RV 20.57.306-98 | air overpressure: up to 10 kgs/sq сm | 1,000; 2,0002,0002,000 |
| 2.12 Trials for waterproofness | GOST RV 20.57.306-98 | water overpressure: up to 5 kgs/sq сm |  |
| 2.13 Trials for exposure to atmospheric precipitations (rain) | GOST RV 20.57.306-98 | rain rate: from 5 mm/min to 10 mm/min | 1,000; 2,4002,4002,400 |
| 2.14 Trials for exposure to lower humidity | GOST RV 20.57.306-98 | temperature: from 10 ºС to 60 ºС; relative humidity: up to 10%; pressure: up to 5 mm Hg | 1,000; 2,0502,3001,870 |
| 2.15 Trials for exposure to higher or lower temperatures in case of vacuum | GOST RV 20.57.306-98 | up to 5 mm Hg; temperature: from – 75 ºС to 300 ºС | 1,000; 1,0001,0001,000 |
| 2.16 Trials for heatresistance in case of simulated zero-gravity | GOST RV 20.57.306-98 | up to 5 mm Hg; temperature: from – 75 ºС to 300 ºС | 1,000; 1,0001,0001,000 |
| 2.17 Trials for exposure to solar radiation in case of vacuum | GOST RV 20.57.306-98 | density of IR-radiation flux: up to 1,200 W/sq m; density of UV-radiation flux: up to 68 W/sq m within from 0.32 µm to 0.4 µm; pressure: up to 5 mm Hg | 1,000; 2,0502,3001,870 |
| 2.18 Trials for exposure to dew and internal icing | GOST RV 20.57.306-98 | temperature: from – 40 ºС to 60 ºС; relative humidity: up to 95%; pressure: up to 5 mm Hg | 1,000; 2,0502,3001,870 |
| 2.19 Trials of package for resistance to higher temperature | GOST RV 20.57.310-98 | temperature: from – 75 ºС to 100 ºС pressure: up to 5 mm Hg | 1,000; 2,0502,3001,870 |
| 3 Assessment of electrical insulation for compliance with given requirements | GOST RV 20.39.308-98, specifications for products, trial programmes and procedures, GOST RV 20.57.310-98 |  |  |
| 3.1 Check of electrical resistance of insulation | GOST RV 20.57.310-98 | up to 1,000 МΩ; voltage: from 10 V to 2,500 V |  |
| 3.2 Check of electrical strength of insulation | GOST RV 20.57.310-98 | up to 1 kV |  |
| 4 Check of transient resistance | specifications for products, drawings, check procedures | from 1 µΩ to 2 МΩ |  |